CS  M51A and EE  M16 Spring 2015  Section  1

Logic Design of Digital Systems

Dr. Yutao He

Verilog Lab #2 - Design of Combinational Systems

Due:  May 13rd, 2015

(1)     Name:

Yu                         Dau-Po

Last                            First

Student ID:	504451468

Signature:	Dau-Po Yu

(2)     Name:

Lin                         Jennifer

Last                            First

Student ID:  	904299916

Signature:    Jennifer Lin

Date:    5/13/2015

Result Correctness Creativity Report

Total Score

Verilog Lab #2  Project  Requirement

Dr. Yutao He

Due: 5/13/2015

1    Objectives

The second project is to build a combinational circuit for a real-world appli- cation by applying systematically the concepts and techniques  covered in the class, and walking through  the whole cycle of designing digital systems, from the high-level specification to the final implementation. It basically consists of two steps:(1)  the  pencil-and-paper design,  and  (2)  the  implementation with Verilog by means of the CAD software Vivado.

2    Project  Description

2.1     The High-Level Specification

The  circuit  to be built  is a BCD-to-seven-segment display  decoder  that is used extensively to drive LED (Light-Emitting Diode)-based  display devices such as billboards.

The structure of a seven-segment LED device and its interface with the decoder  to be designed  are  shown  in Figure  1.   It is easily  seen that any decimal number  from zero to nine can be displayed  with this device simply

by turning some segments on (shaded), while leaving others oﬀ (unshaded).

a

f         b

g                             1

0

1 e         c                        0

d

b   0

1

a   1                        a

0            f                        f         b

BCD-to-7-Segment

Display Decoder

g

1                 1                        g

2

3 	e   0                  e         c d   1

c   1                        d

unshaded shaded

The circuit to be designed

Display of the digit 5

Figure  1: The Structure of a Seven-Segment Display Device

The complete display  patterns for one-digit decimal numbers  are speci- fied in Table  1.

| Decimal No. 0 | a ON | b ON | c ON | d ON | e ON | f ON | g OFF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | OFF | ON | ON | OFF | OFF | OFF | OFF |
| 2 | ON | ON | OFF | ON | ON | OFF | ON |
| 3 | ON | ON | ON | ON | OFF | OFF | ON |
| 4 | OFF | ON | ON | OFF | OFF | ON | ON |
| 5 | ON | OFF | ON | ON | OFF | ON | ON |
| 6 | ON | OFF | ON | ON | ON | ON | ON |
| 7 | ON | ON | ON | OFF | OFF | OFF | OFF |
| 8 | ON | ON | ON | ON | ON | ON | ON |
| 9 | ON | ON | ON | ON | OFF | ON | ON |

Table  1: The Seven-Segment Display Logic Specification

Given  the  above specification,  in this  project  you are to build  a circuit which takes a decimal digit, decodes it according  to the specified logic, and generates  the corresponding  outputs.  In  addition, your  circuit must also meet the following requirements:

1.  Encoding  scheme  of  inputs:   You  must  use  the BCD  (Binary- Coded-Decimal)  code to encode one decimal  digit  as specified in Ta- ble 2:

2.  Encoding scheme of outputs:  The on/oﬀ encoding scheme of out- puts  is specified in Table  3.

3.  Implementation:  The system must be implemented with a two-level

NAND-NAND network.

4.  Cost:  Let  us define the cost of the  system  as the  number  of NAND

gates in the circuit.  The system must be designed with minimal  cost.

3    Report  Outline

Each  team is required  to submit only  one report that provides  complete documentation of your project including the detailed design worksheets.  As in all technical writing, its purpose is to communicate your work with your

| Decimal No. 0 | BCD Code 0000 |
| --- | --- |
| 1 | 0001 |
| 2 | 0010 |
| 3 | 0011 |
| 4 | 0100 |
| 5 | 0101 |
| 6 | 0110 |
| 7 | 0111 |
| 8 | 1000 |
| 9 | 1001 |

Table  2: The Input Encoding  Scheme

Segment State    Binary Code

ON                            1

OFF                           0

Table  3: The Output Encoding  Scheme

colleagues in an  eﬃcient and  professional  way so that your  design can  be continuously  upgraded  and  maintained  even  if you  are  no longer  around. As a result, the report should  be clear,  concise, and  complete and  should contain the following parts:

(1)  Title  Page

It  is provided  and  you  just  need  to fill in  your  information in  the blanks.

(2)  Abstract

This is the brief high-level description of the project. (3)  The Switching Functions of the Circuit

It is part of the design work for you to obtain  the binary-level specifica-

tion for the function  of the circuit in the form of switching expressions and the schematic.  This section should present both minimal switching expressions in NAND-NAND  form and the schematic  of the circuit.

(4)  The Verilog Code of the Circuit

The Verilog file (with extension .v) you write is the implementation of the  circuit.   You should include its printout in your report with  your names and student IDs on it.

(5)  The Simulation  Result

You have to demonstrate that your implementation behaves  as speci- fied by showing the correct simulation result.  In particular, the result (in  the  forms of Timing  Diagram)  should  include  the  displays  for a complete  set of decimal numbers  (from zero to nine).  Include in your report the Timing  Diagram  with enough information on it so that one of your  colleagues  who does not  know anything about your  project could understand  easily which function  you are trying  to implement by just reading  the Timing  Diagram.

(6)  The Design Review

This section summarizes  your experiences and lessons throughout the project.  It should be no more than one page and  may include topics such as what you have  learned,  problem  encountered during  the im- plementation and  the solutions  you came up with, the approach  you used,  the most  important aspects of the project for you,  where you spent most of your time, and suggestions  you would like to make.

(7)  Team  Member Contributions

Teamwork  requires  that each member  be responsible  and  assume  an relatively equal share of workload.  In this section, a detailed descrip- tion on each member’s responsibility  and contribution should be pre-

sented  clearly,  including  an  estimate  of percentage  of eﬀorts  on  the

project and a summary  list of each member  in the project.

Each member requires to review and sign the final report on the cover page.

(8)  Appendix - The Detailed  Design Worksheet

This  section must  include  the complete set of worksheets from  the pencil-and-paper design.   Please  scan  and  combine  them with other parts of your report.  Hand-written form is perfectly fine yet it must include:

8.1  inputs and outputs of the system.

8.2  encoding schemes of inputs and output.

8.3  truth tables.

8.4  minimization  procedure  by  means  of either  K-map  or  Quine- McCluskey method.

8.5  transformation procedure.

8.6  final minimal expressions of the output functions in terms of both switching  expressions and the schematic.

4    Project  Submission

You  should  submit one zipped  file named  with ”Txx.zip” via  the on-line submission link that will be avalable  by 5/10.  The zipped file should consist of three  separate  files:

1.  The pdf file of your report.  It must be named  as ”Txx.pdf” where xx is your Team  ID assigned to you, that is, your report should be called Txx.pdf;

2.  The  Verilog file of your  circuit  implementation.   It  should  be named as csm51a  proj2.v.

3.  The testbench file. It should be named  as csm51a  proj2  tb.v.

5    Project  Deadline

The  report  is due  by  midnight  (11:59:59pm)  on  May  13rd  (Wednesday),

2015.  The  deadline  must be observed  strictly and  late submissions  will be subject to penalty.

Abstract

For this project, the circuit we implemented is a BCD-to-seven-segment display decoder that could display numbers zero through nine by lightening up certain segments of the device. The circuit takes in four inputs that encode decimal numbers zero through nine using BCD code, and outputs a through g uses the on/off encoding scheme specified in Table 3. The system is implemented with a two-level NAND-NAND network.

Switching Functions

Switching functions of outputs a to g:

a= [x3’x1’(x2x0)’(x2’x0’)’]’

b= [x2(x1’x0’)’(x1x0)’]’

c= (x2’x1x0’)’

d= [x3’(x2x1’x0)’(x2x1x0’)’(x2’x1)’(x2’x0’)’]’

e= [(x2’x0’)’(x1x0’)’]’

f= [x3’(x1’x0’)’(x2x1’)’(x2x0’)’]’

g= [(x3x1’)’(x2’x1)’(x2x0’)’(x2x1’)’]’

Verilog Code

module csm51a_proj2(x3, x2, x1, x0, a, b, c, d, e, f, g);

input x3, x2, x1, x0;

output a, b, c, d, e, f, g;

assign a= ~( (~x3) & (~x1) & ~(x2 & x0) & ~(~x2 & ~x0) );

assign b= ~( (x2) & ~(~x1 & ~x0)& ~(x1 & x0) );

assign c= ~( (x1) & (~x0) & (~x2) );

assign d= ~( (~x3) & ~(x2 & ~x1 & x0) & ~(x2 & x1 & ~x0) & ~(~x2 & x1) & ~(~x2 & ~x0) );

assign e= ~(~(~x2 & ~x0) & ~(x1 & ~x0));

assign f= ~( (~x3) & ~(~x1 & ~x0) & ~(x2 & ~x1) & ~(x2 & ~x0));

assign g= ~(~(x3 & ~x1) & ~(~x2 & x1) & ~(x1 & ~x0) & ~(x2 & ~x1));

endmodule

Testbench

module csm51a_proj2_tb;

wire t_1, t_2, t_3, t_4, t_5, t_6, t_7;

reg t_a, t_b, t_c, t_d;

csm51a_proj2 my_gate( .x3(t_a), .x2(t_b), .x1(t_c), .x0(t_d), .a(t_1), .b(t_2), .c(t_3), .d(t_4), .e(t_5), .f(t_6), .g(t_7));

initial

begin

$monitor(t_a, t_b, t_c, t_d, t_1, t_2, t_3, t_4, t_5, t_6, t_7);

t_a = 1'b0;

t_b = 1'b0;

t_c = 1'b0;

t_d = 1'b0;

#5

t_a = 1'b0;

t_b = 1'b0;

t_c = 1'b0;

t_d = 1'b1;

#5

t_a = 1'b0;

t_b = 1'b0;

t_c = 1'b1;

t_d = 1'b0;

#5

t_a = 1'b0;

t_b = 1'b0;

t_c = 1'b1;

t_d = 1'b1;

#5

t_a = 1'b0;

t_b = 1'b1;

t_c = 1'b0;

t_d = 1'b0;

#5

t_a = 1'b0;

t_b = 1'b1;

t_c = 1'b0;

t_d = 1'b1;

#5

t_a = 1'b0;

t_b = 1'b1;

t_c = 1'b1;

t_d = 1'b0;

#5

t_a = 1'b0;

t_b = 1'b1;

t_c = 1'b1;

t_d = 1'b1;

#5

t_a = 1'b1;

t_b = 1'b0;

t_c = 1'b0;

t_d = 1'b0;

#5

t_a = 1'b1;

t_b = 1'b0;

t_c = 1'b0;

t_d = 1'b1;

#5

t_a = 1'b1;

t_b = 1'b0;

t_c = 1'b1;

t_d = 1'b0;

#5

t_a = 1'b1;

t_b = 1'b0;

t_c = 1'b1;

t_d = 1'b1;

#5

t_a = 1'b1;

t_b = 1'b1;

t_c = 1'b0;

t_d = 1'b0;

#5

t_a = 1'b1;

t_b = 1'b1;

t_c = 1'b0;

t_d = 1'b1;

#5

t_a = 1'b1;

t_b = 1'b1;

t_c = 1'b1;

t_d = 1'b0;

#5

t_a = 1'b1;

t_b = 1'b1;

t_c = 1'b1;

t_d = 1'b1;

end

endmodule

Simulation Result

Design Review

This project enlightens me how to solve a real-world question. Before this project, I would not know how to translate and apply what we learn in class into real-world application. However, after this assignment, I understand how to approach a question in a different perspective, and further solve it with what we learn in class.

During this project, we spent some time writing, checking, and double-checking the minimum minterm expression. This step is extremely important, since we are required to minimize the cost, and the process after it would be in vain if this step is wrong. Additionally, testbench may not detect the errors here, so we were really careful about this step. Afterwards, we went smoothly in implementing and stimulating the modules.

Team Member Contribution

The workload of this project is equally shared between us. Rather than dividing and assigning who should do what part of this project, both of us did most of the work and checked on each other’s work to make sure that we’re on the right track.

Appendix